This comprehensive dataset is compiled for your app development, covering the 71.6 km Pink Line (Line 7) including the Phase 4 Ring extension (operational March 2026).

Line 7 Technical Overview

- Total Stations: 46

- Total Length: ~71.6 km

- Track Gauge: 1435 mm (Standard Gauge)

- Signalling: CBTC (Communication Based Train Control)

- Rolling Stock: 6-Coach Driverless enabled (Unattended Train Operation)

Master Station & Crossover List (Sequential Loop)

Chainages (

) are in metres. Reversal/Crossover points are listed where they occur between or at stations.

| Station Name | Center CH (m) | Crossover Type | Crossover CH (m) | Turnout Size (L) | Purpose |
| --- | --- | --- | --- | --- | --- |
| Majlis Park | 0 | Double | (-) 120 | 75 m | Terminal Reversal / Depot Entry |
| Azadpur | 1,550 | - | - | - | Interchange (Yellow/Magenta) |
| Shalimar Bagh | 3,009 | - | - | - | - |
| Netaji Subhash Place | 4,244 | Single | 4,380 | 40 m | Emergency Reversal |
| Shakurpur | 5,215 | Double | 5,350 | 75 m | Operational Flexibility |
| Punjabi Bagh West | 6,685 | - | - | - | Halt (Interchange via bridge) |
| ESI - Basaidarapur | 8,411 | - | - | - | - |
| Rajouri Garden | 9,324 | - | - | - | Interchange (Blue) |
| Mayapuri | 10,571 | - | - | - | - |
| Naraina Vihar | 11,886 | - | - | - | - |
| Delhi Cantt. | 13,312 | - | - | - | - |
| D.D. South Campus | 16,468 | Single | 16,620 | 40 m | Short-loop Terminus Point |
| Moti Bagh | 17,589 | - | - | - | - |
| Bhikaji Cama Place | 18,914 | - | - | - | - |
| Sarojini Nagar | 19,960 | - | - | - | - |
| Dilli Haat - INA | 20,992 | - | - | - | Interchange (Yellow) |
| South Extension | 22,152 | - | - | - | - |
| Lajpat Nagar | 23,589 | Single | 23,740 | 40 m | Peak-hour Turnback |
| Vinobapuri | 24,828 | - | - | - | - |
| Ashram | 25,848 | - | - | - | Smallest station footprint |
| SKK - Nizamuddin | 27,324 | - | - | - | ISBT / Railway Hub |
| Mayur Vihar - I | 30,537 | - | - | - | Interchange (Blue) |
| Shree Ram Mandir | 31,377 | Double | 31,520 | 75 m | Major Operational Reversal |
| Trilokpuri-Sanjay Lake | 32,520 | - | - | - | - |
| E. Vinod Nagar-MV II | 33,189 | - | - | - | - |
| Mandawali-W. Vinod Nagar | 33,640 | - | - | - | - |
| I.P. Extension | 34,483 | Double | 34,650 | 75 m | Depot Entry (Vinod Nagar) |
| Anand Vihar ISBT | 35,901 | - | - | - | Hub (Blue/Railway/RRTS) |
| Karkarduma | 36,799 | - | - | - | Interchange (Blue) |
| Karkarduma Court | 37,823 | - | - | - | - |
| Krishna Nagar | 38,450 | - | - | - | - |
| East Azad Nagar | 39,235 | - | - | - | - |
| Welcome | 40,131 | - | - | - | Interchange (Red) |
| Jafrabad | 41,153 | - | - | - | - |
| Maujpur-Babarpur | 42,112 | Scissors | 42,280 | 110 m | Loop/Branch Junction |
| Yamuna Vihar | 43,190 | - | - | - | - |
| Bhajanpura | 44,150 | - | - | - | - |
| Khajuri Khas | 45,450 | - | - | - | - |
| Sonia Vihar | 47,000 | Single | 47,160 | 40 m | River Bridge Safety Reversal |
| Soorghat | 48,600 | - | - | - | - |
| Jagatpur-Wazirabad | 50,200 | Single | 50,340 | 40 m | Loop Regulation |
| Jharoda Majra | 52,100 | - | - | - | - |
| Burari | 54,500 | - | - | - | - |
| (Back to Majlis Park) | ~71,560 | - | - | - | Loop Complete |

Branch Line Extension (Linear)

Branching from Maujpur-Babarpur.

| Station Name | Center CH (m) | Crossover Type | Crossover CH (m) | Turnout Size (L) |
| --- | --- | --- | --- | --- |
| Gokulpuri | 43,212 | - | - | - |
| Johri Enclave | 44,412 | - | - | - |
| Shiv Vihar | 45,312 | Scissors | 45,460 | 110 m |

Logic Definitions for App Creation

- Crossover Scissors (110m): High-speed switches allowing simultaneous "X" crossings. Used where the branch separates from the loop.

- Crossover Double (75m): Two back-to-back switches. Essential for terminal turn-backs at Majlis Park and I.P. Ext.

- Crossover Single (40m): Simplest diagonal link. Use these in your app for "Short Loop" service scenarios (e.g., trains terminating early at South Campus).